2210 PROM Load/Dump Program code updates

prom_1.31

_________

V1.31 is a service level of the PROM Load/Dump Program revision level 1.30

for 2210 hardware models 1S4, 1S8, 1U4 and 1U8.

IBM 2210 PROM Load/Dump Program Installation Instructions

2210 PROM LOAD/DUMP PROGRAM

___________________________

The PROM Load/Dump Program is the program that performs boot operations and

diagnostics on the 2210. This program is kept in Boot Flash and can be

replaced as necessary to support new adapters or install corrective service.

DETERMINING THE PROM LOAD/DUMP PROGRAM VERSION LEVEL

The first step is to determine if the 2210 already has the needed version

of the PROM Load/Dump Program.  A scenario for determining the PROM

Load/Dump Program revision level is shown below.

1.  At the OPCON prompt (*):

Enter TALK 5 to access the Monitor command environment

CGW Operator Console   (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

A.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

B.  You should something like the following on the Configuration

Information screen:

Boot ROM version  V1.30.

At the + prompt press the CTRL and P keys simultaneously to return to the

OPCON prompt (*).

+ CTRL C

*

3.  If the version number is equal to the desired version of the PROM

Load/Dump Program you do not have upgrade it.

LOADING THE PROM LOAD/DUMP PROGRAM

The 2210 PROM Load/Dump Program is kept in Boot Flash which is only acces-

sible by the PROM Load/Dump Program.  There are various ways to load the PROM

Load/Dump Program into Boot Flash.  Once it is loading on the router

and executes, it will load itself into Boot Flash.

THE 2210 MUST NOT BE POWERED OFF DURING EXECUTION.  THIS COULD CAUSE BOOT

FLASH CORRUPTION.

The various ways to load the 2210 PROM Load/Dump Program into the 2210 DRAM

are shown below.  The following scenarios are examples of some of the ways of

loading the PROM Load/Dump Program into Boot Flash.

LOADING TECHNIQUE                       LOADING STEPS

"Loading via zmodem over the 2210       PC via service port>DRAM>Boot Flash

Service Port"

"Loading Using TFTP get"                Server via LAN or PPP link>IBD>D

RAM>Boot Flash

"Loading using Boot Manual over the     Server via LAN or PPP link>DRAM> Boot

LAN or PPP link"                        Flash

Loading via zmodem over the 2210 Service Port

This scenario loads the 2210 PROM Load/Dump Program into DRAM and initiates

its execution.  The PROM Load/Dump Program will load itself into Boot Flash.

NOTE:  This example assumes that the PROM Load/Dump Program has been loaded

onto the PC's c:\promcode directory as prom_x.yy and that the PC is connected

to the 2210's service port.

Perform the following steps while viewing the 2210 console via a service-

port-attached PC running a terminal emulator.  The PC can be locally attached

or remotely attached via a modem.

1.  At the OPCON prompt (*), enter RELOAD to initiate the reloading of the

2210.

* RELOAD

Are you sure you want to reload the gateway? (Yes or [NO]): Y

The PC console will display something similar to the following example:

PROM Load/Dump Program * Revision:  1.30  *

Copyright  1985-1996 Proteon, Inc.  All rights reserved

Loading using IBD Load Image "MRS.XXX"   (XXX is Software Exten-

sion)

2.  While the machine is reloading, and ------- appears on the console, do

the following:

o   Press the "CTRL and C" keys simultaneously to terminate reloading and

put the 2210 into Boot Monitor state.  The (>) prompt will be dis-

played, for example:

--------- Ctrl c

>

The PC console will display something similar to the following

example:

Bootstrap Monitor  V1.30

Copyright  1990-1996  Proteon, Inc

o   At the (>) prompt, enter ZB to initiate loading the new PROM

Load/Dump Program over the service port using zmodem.

> ZB

o   The following dialogue will load the new PROM Load/Dump Program into

DRAM and initiate its execution. The PROM Load/Dump Program will load

itself into Boot Flash.

-   From the 2210

Are you sure you want to load via the console ? Y

Okay, GO!!

-   Switch to the Upload Screen of your terminal emulator (refer to

your terminal emulator user manual).

-   Select ZMODEM as the upload protocol

--  Enter your module name, for example, "c:\promcode\prom_x.yy"

-   From the 2210

Load SUCCEEDED

Starting at 1200000

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

Resetting router, please wait ...

PROM Load/Dump Program * Revision:  x.yy *

Copyright 1996-1997 IBM Corp.

Copyright 1985-1996 Proteon, Inc. All rights reserved

Diagnostic code version 2.32

Copyright 1995-1997 IBM Corp.

No valid boot or bootp records found, attempting IBD load

Loading using IBD Load Image "MRS.XXX"   (XXX is Software

Extension)

----------------

.......

The new version of the 2210 PROM Load/Dump Program, x.yy, is now active and

loaded into Boot Flash.

The 2210 returns to the OPCON prompt (*).

If you want to verify that the new PROM Load/Dump Program image is loaded

into Boot Flash, use the following scenario.

1.  At the OPCON prompt (*):

Enter TALK 5 to access the Monitor command environment

CGW Operator Console   (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

A.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

B.  You should see the following on the Configuration Information screen:

Boot ROM Version  Vx.yy

Loading Using TFTP get

This scenario assumes the 2210 is operational and has network access to a PC

or workstation running a TFTP server.  It provides the least network outage

of the methods listed in this document, and performs these tasks:

o   Loads the 2210 PROM Load/Dump Program into the IBD (this is temporary and

you should delete it once it has been stored into Boot Flash)

o   Initiates its execution which will allow the PROM Load/Dump Program to

load itself into Boot Flash

o   Erases the PROM Load/Dump Program from the IBD

NOTE:  You will lose your session when the PROM Load/Dump Program begins

loading if you have telneted into the 2210. The 2210 will automatically

reload the MRS code according to your booting specification once the PROM

Load/Dump Program is loaded into Boot Flash.  At that time you can telnet

back to the 2210

IMPORTANT:- If you do not have boot specifications, or your boot specifica-

tions fail, the 2210 will boot using the first entry in the IBD.  Conse-

quently, the PROM load MUST NOT be the first entry in the IBD.

________

The detailed steps follow:

1.  Install your PROM Load/Dump Program into a PC or workstation running a

TFTP server that is reachable by the 2210.  Ensure that the TFTP server

code is running before continuing.

2.  Ensure that the 2210 is configured such that it can communicate with the

PC running the TFTP server.

The following steps are performed while viewing the 2210 console via a local

or remote service port connection or a telnet session:

3.  At the OPCON prompt (*) enter TALK 6 to access the Config> command envi-

ronment.

* TALK 6

Config>

4.  At the CONFIG  prompt (Config>) enter BOOT to access the Boot config

command environment.

Config> BOOT

Boot config>

5.  At the Boot config> prompt, enter LIST IBD.  This will show you what is

already stored in the IBD.  If you need to make room for the PROM

Load/Dump Program, you can erase any unneeded files in the IBD using the

ERASE command.

NOTE:  The Boot Code will be loaded into the first empty IBD bank.

Ensure that there is a valid bootable code image left in the IBD ahead of

the first empty bank.

You will need 1 bank for this load operation.

To erase an old load or configuration file enter ERASE and the filename.

Boot config> ERASE FILENAME

6.  At the Boot "config>" prompt, enter TFTP GET.  This initiates the file

transfer of the load module.

You will be prompted for the following information:

o   Local filename.  For the local filename, enter the filename of the

new PROM Load/Dump Program (prom_x.yy).

o   Remote host.  For the remote host, enter the IP address of the server

(this example assumes that the remote host IP address is 1.1.1.101).

o   Host filename.  For the Host filename, specify the entire path and

filename on the host machine (this example assumes that the PROM

Load/Dump Program is on the "/promcode" directory).

Boot config> TFTP GET

Local filename [CONFIG]?  IBD/PROM_x.yy

Remote host [0.0.0.0]? 1.1.1.101

Host filename [030E4111.cfg] /PROMCODE/PROM_x.yy

TFTP transfer complete, status: OK

Boot config>

7.  At the Boot "config>" prompt, enter LIST IBD.  The new PROM Load/Dump

Program, prom_x.yy should be listed.

Boot config> LIST IBD

The PROM Load/Dump Program, prom_x.yy, is now loaded into the IBD.

The next step is to load the PROM Load/Dump Program into Boot Flash.  This

can only be accomplished by loading the PROM Load/Dump Program into DRAM and

allowing it to execute.

1.  At the Boot config prompt, enter the following to load the new PROM

Load/Dump Program into Boot Flash:

Boot config> LOAD LOCAL

Loadname: IBD/PROM_x.yy

Are you sure you want to reload the gateway?(Yes or [No]): Y

PROM Load/Dump Program * Revision:  1.30 *

Copyright 1985-1996 Proteon, Inc. All rights reserved

Loading using IBD Load Image "prom_x.yy"

---

Starting at 1200000

NOTE:  If you have telneted into the 2210 you will lose your session at

this point.  Once the 2210 has re-booted (about 2 minutes), you can

telnet back to the 2210 and continue with "2" below.

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

Resetting router, please wait ...

PROM Load/Dump Program * Revision:  x.yy *

Copyright 1996-1997 IBM Corp.

Copyright 1985-1996 Proteon, Inc. All rights reserved

Diagnostic code version 2.32

Copyright 1995-1997 IBM Corp.

Loading using IBD Load Image "MRS.XXX"   (XXX is Software Exten-

sion)

----------------

2.  If you want to verify that the new PROM Load/Dump Program image is loaded

into Boot Flash, use the following scenario.

A.  At the OPCON prompt (*):

o   Enter TALK 5 to access the Monitor command environment (+).

* TALK 5

+

B.  At the MONITOR prompt (+):

1)  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

2)  You should see the following on the Configuration Information

screen:

Boot ROM Version  Vx.yy

The final step is to erase the PROM Load/Dump Program from the IBD where it

was stored earlier.  At the + prompt, press the CTRL and P keys simultane-

ously to get to the OPCON prompt (*).

+ CTRL C

*

At the OPCON prompt (*) enter TALK 6 to access the Config> command environ-

ment.

* TALK 6

Config>

At the CONFIG  prompt (Config>) enter BOOT to access the Boot config command

environment.

Config> BOOT

Boot config>

To erase the temporary copy of the PROM Load/Dump Program, prom_x.yy, enter

ERASE and the filename at the "Boot config>" prompt.

Boot config> ERASE prom_x.yy

The new version of the 2210 PROM Load/Dump Program, x.yy, is now active and

loaded into Boot Flash.

Loading using Boot Manual over the LAN or PPP link

This scenario assumes that the 2210 is connected via a LAN or PPP link to a

PC or workstation running a TFTP server and local or remote Service Console

via the service port.

This scenario will load the PROM Load/Dump Program into DRAM and initiate its

execution.  The PROM Load/Dump Program will load itself into Boot Flash.

NOTE:  You must enter the responses that correspond to your environment.

1.  Install the PROM Load/Dump Program into a PC or workstation running a

TFTP server that is reachable by the 2210.  Ensure that the TFTP server

code is running before continuing.

The following steps are performed while viewing the 2210 console via a local

or remote service port connection.

2.  At the OPCON prompt (*), enter RELOAD to initiate the reloading of the

2210.

* RELOAD

Are you sure you want to reload the gateway? (Yes or [NO]): Y

3.  While the machine is reloading (---- appears on the console),

Press the "CTRL and C" keys simultaneously to terminate reloading and

put the 2210 into Boot Monitor state (the (>) prompt will be dis-

played).

--------- CTRL C

>

4.  At the (>) prompt, enter BM to initiate the Boot Manual process.

> BM

5.  The following dialogue will load the new PROM Load/Dump Program into DRAM

and initiate its execution. The PROM Load/Dump Program will load itself

into Boot Flash

NOTE:  YOU MUST SPECIFY THE PARAMETERS THAT MATCH YOUR ENVIRONMENT.

___________________________________________________________

Device types available:

IBD

Ethernet

WAN

Device type: ETHERNET

Interface IP address [ ]: 1.1.1.1

IP mask [FFFFFF00]: FFFFFF00

Boot from host: 1.1.1.101

Boot file name: /PROMCODE/PROM_x.yy

Using Ethernet at (     0, 0).

Trying host 1.1.1.101

file /u/diagnostic/prom_x.yy

loading

...........

Starting at 1200000

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

PROM Load/Dump Program * Revision:  x.yy *

Copyright  1996-1997 IBM Corp

Copyright  1985-1996 Proteon, Inc.  All rights reserved

Diagnostic Code Version 2.32

Copyright  1995-1997 IBM Corp

......

Loading using IBD Load Image "MRS.XXX"        (XXX is Software Exten-

sion)

----------------

The new version of the 2210 PROM Load/Dump Program, x.yy, is now active and

loaded into Boot Flash.

If you want to verify that the new PROM Load/Dump Program image is loaded

into Boot Flash, use the following scenario.

1.  At the OPCON prompt (*):

Enter TALK 5 to access the Monitor command environment (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

A.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

B.  You should see the following text on the Configuration Information

screen:

Boot ROM Version  Vx.yy