IBM 2210 PROM Load/Dump Program Installation Instructions

PREREQUISITES

_____________

Prior to utilizing your adapter, you  must ensure that you have the correct

PROM Load/Dump Program.

The following section describes how to perform the verification.

IBM 2210 PROM LOAD/DUMP PROGRAM

_______________________________

The PROM Load/Dump Program is the program that performs boot operations and

diagnostics  on the IBM 2210. This program is kept in Boot Flash and can be

replaced as necessary to support new adapters or  install  corrective  ser-

vice.

IN  ORDER  TO USE THE ISDN BRI ADAPTER, ISDN PRI ADAPTER, 25MBPS ATM INTER-

FACE ADAPTER OR THE 4 OR 8 PORT WAN CONCENTRATION ADAPTERS, THE IBM  2210'S

PROM LOAD/DUMP PROGRAM MUST BE AT VERSION 2.20 OR HIGHER.  THE ADAPTERS AND

THE  PROM  LOAD/DUMP  PROGRAM  CAN  BE  INSTALLED AT DIFFERENT TIMES BUT AS

STATED ABOVE THE PROM LOAD/DUMP PROGRAM MUST BE AT VERSION 2.20  OR  HIGHER

BEFORE THE ADAPTERS CAN BE USED.

DETERMINING THE PROM LOAD/DUMP PROGRAM VERSION LEVEL

____________________________________________________

The first step is to determine if the IBM 2210 already has a version of the

PROM  Load/Dump Program that will work with the adapters.  This section de-

scribed how to determine that version.

Following is the scenario for determining the PROM  Load/Dump  Program  re-

vision level.

1.  At the OPCON prompt (*):

a.  Enter TALK 5 to access the Monitor command environment

CGW Operator Console   (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

a.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

b.  You should see the following or higher Version on the Configuration

Information screen:

Boot ROM version  V2.20.

At  the  + prompt press the CTRL and P keys simultaneously to return to

the OPCOM prompt (*).

+ CTRL C

*

3.  If the above version number is greater than or equal to V2.20, the PROM

Load/Dump Program level is valid for the adapter so you do not have up-

grade it.

LOADING THE PROM LOAD/DUMP PROGRAM

__________________________________

The IBM 2210 PROM Load/Dump Program is kept in Boot Flash which is only ac-

cessible by the PROM Load/Dump Program.   The only way  to  load  the  PROM

Load/Dump  Program  into  Boot Flash is to load the PROM Load/Dump Program,

prom_2.00, from the diskette included with this shipment into the IBM  2210

DRAM  and  allow it to execute.  During execution, it will load itself into

Boot Flash.

There are various ways to load the IBM 2210 PROM Load/Dump Program into the

IBM 2210 DRAM.  The methods listed below are described in this document.

LOADING TECHNIQUE                   STEPS OF LOAD

1.  Loading using "zmodem"              PC via service port>DRAM>Boot Flash

2.  Loading using "tftp get"            Server via LAN or PPP link>IBD>

DRAM>Boot Flash

3.  Loading using "boot manual"         Server via LAN or PPP link>

DRAM>Boot Flash

The following scenarios are examples of some of the  ways  of  loading  the

PROM Load/Dump Program into Boot Flash.

LOADING THE PROM LOAD/DUMP PROGRAM VIA ZMODEM OVER THE IBM 2210 SERVICE

_______________________________________________________________________

PORT

____

This  scenario  will load the IBM 2210 PROM Load/Dump Program into DRAM and

initiate its execution.  The PROM Load/Dump Program will load  itself  into

Boot Flash.

NOTE:  THIS EXAMPLE ASSUMES THAT THE PROM LOAD/DUMP PROGRAM HAS BEEN LOADED

ONTO THE PC'S C:\PROMCODE DIRECTORY AS PROM_2.20 AND THAT THE  PC  IS  CON-

NECTED TO THE IBM 2210'S SERVICE PORT.

The  following steps are performed while viewing the IBM 2210 console via a

service-port-attached PC running a terminal emulator.   The PC can  be  lo-

cally attached or remotely attached via a modem.

1.  At  the OPCON prompt (*), enter RELOAD to initiate the reloading of the

IBM 2210.

* RELOAD

Are you sure you want to reload the gateway? (Yes or [NO]): Y

The PC console will display the following:-  (EXAMPLE)

PROM Load/Dump Program * Revision:  1.10  *

Copyright  1985-1996 Proteon, Inc.  All rights reserved

Loading using IBD Load Image "MRS.XXX"   (XXX is S/W Extension)

2.  While the machine is reloading (------- appears on the console),

o   Press the "CTRL and C" keys simultaneously to  terminate  reloading

and  put  the IBM 2210 into Boot Monitor state (the (>) prompt will

be displayed).

--------- CTRL C

>

The PC console will display the following:-

Bootstrap Monitor  V1.10

Copyright  1990-1996  Proteon, Inc

3.  At the (>) prompt, enter ZB to initiate loading the new PROM  Load/Dump

Program over the service port using zmodem.

> ZB

4.  The  following  dialogue  will load the new PROM Load/Dump Program into

DRAM and initiate its execution. The PROM Load/Dump Program  will  load

itself into Boot Flash.

o   From the IBM 2210

Are you sure you want to load via the console ? Y

Okay, GO!!

o   Switch  to  the  Upload  Screen of your terminal emulator (refer to

your terminal emulator user manual).

o   Select ZMODEM as the upload protocol

-   Enter your module name E.G.    C:\PROMCODE\PROM_2.20

o   From the IBM 2210

Load SUCCEEDED

Starting at 1200000

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

PROM Load/Dump Program * Revision:  2.20 *

Copyright  1985-1996 Proteon, Inc.  All rights reserved

......

Loading using IBD Load Image "MRS.XXX"     XXX is S/W  Exten-

sion)

----------------

.......

The new version of the IBM 2210 PROM Load/Dump Program, 2.20, is now active

and loaded into Boot Flash.

The IBM 2210 returns to the OPCOM prompt (*).

If  you  want to verify that the new PROM Load/Dump Program image is loaded

into Boot Flash, use the following scenario.

1.  At the OPCON prompt (*):

a.  Enter TALK 5 to access the Monitor command environment

CGW Operator Console   (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

a.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

b.  You should see  the  following  on  the  Configuration  Information

screen:

Boot ROM Version  V2.20

LOADING THE PROM LOAD/DUMP PROGRAM USING TFTP GET

_________________________________________________

This scenario assumes the IBM 2210 is operational and has network access to

a  PC  or workstation running a TFTP server.  It provides the least network

outage of the methods listed in this document, and does the following:

o   Loads the IBM 2210 PROM Load/Dump Program into the IBD (this is  tempo-

rary and you should delete it once it has been stored into Boot Flash)

o   Initiates  its execution which will allow the PROM Load/Dump Program to

load itself into Boot Flash

o   Erases the PROM Load/Dump Program from the IBD

NOTE: YOU WILL LOSE YOUR SESSION WHEN THE  PROM  LOAD/DUMP  PROGRAM  BEGINS

LOADING IF YOU HAVE TELNETED INTO THE 2210. THE 2210 WILL AUTOMATICALLY RE-

LOAD  THE  MRS  CODE  ONCE  THE  PROM LOAD/DUMP PROGRAM IS LOADED INTO BOOT

FLASH.  AT THAT TIME YOU CAN TELNET BACK TO THE 2210.

The detailed steps are as follows:

1.  Install your PROM Load/Dump Program into a PC or workstation running  a

TFTP  server  that  is  reachable by the IBM IBM 2210.  Ensure that the

TFTP server code is running before continuing.

2.  Ensure that the IBM 2210 is configured such  that  it  can  communicate

with the PC running the TFTP server.

The  following steps are performed while viewing the IBM 2210 console via a

local or remote service port connection or a telnet session:

3.  At the OPCON prompt (*) enter TALK 6 to access the Config> command  en-

vironment.

* TALK 6

Config>

4.  At  the  CONFIG   prompt (Config>) enter BOOT to access the Boot config

command environment.

Config> BOOT

Boot config>

5.  At the Boot config> prompt, enter LIST IBD.  This will show you what is

already stored in the IBD.   If you need to  make  room  for  the  PROM

Load/Dump  Program,  you  can erase any unneeded files in the IBD using

the ERASE command.

You will need 1 bank for this load operation.

To erase an  old  load  or  configuration  file  enter  ERASE  and  the

filename.

Boot config> ERASE FILENAME

6.  At  the Boot "config>" prompt, enter TFTP GET.  This initiates the file

transfer of the load module.

You will be prompted for the following information:

o   Local filename.  For the local filename, enter the filename of  the

new PROM Load/Dump Program (prom_2.20).

o   Remote  host.    For  the  remote host, enter the IP address of the

server (this example assumes that the remote  host  IP  address  is

1.1.1.101).

o   Host  filename.  For the Host filename, specify the entire path and

filename on the host machine (this example assumes  that  the  PROM

Load/Dump Program is on the "/promcode" directory).

Boot config> TFTP GET

Local filename [CONFIG]?  IBD/PROM_2.20

Remote host [0.0.0.0]? 1.1.1.101

Host filename [030E4111.cfg] /PROMCODE/PROM_2.20

TFTP transfer complete, status: OK

Boot config>

7.  At  the  Boot "config>" prompt, enter LIST IBD.  The new PROM Load/Dump

Program, prom_2.20 should be listed.

Boot config> LIST IBD

The PROM Load/Dump Program, prom_2.20, is now loaded into the IBD.

The next step is to load the PROM Load/Dump Program into Boot Flash.   This

can  only  be  accomplished by loading the PROM Load/Dump Program into DRAM

and allowing it to execute.

1.  At the Boot config prompt, enter the following to  load  the  new  PROM

Load/Dump Program into Boot Flash:

Boot config> LOAD LOCAL

Loadname: IBD/PROM_2.20

Are you sure you want to reload the gateway?(Yes or [No]): Y

+--------------------------------------------------+

| If you are telneted into the 2210, you will lose |

| your session and will not receive any of the     |

| following messages. You must re-establish        |

| your session.                                    |

+--------------------------------------------------+

PROM Load/Dump Program * Revision:  1.10 *

Copyright 1985-1996 Proteon, Inc. All rights reserved

Loading using IBD Load Image "prom_2.20"

---

Starting at 1200000

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

PROM Load/Dump Program * Revision:  2.20 *

Copyright  1985-1996 Proteon, Inc.  All rights reserved

......

Loading using IBD Load Image "MRS.XXX"     XXX is S/W Extension)

----------------

2.  If  you  want  to  verify  that the new PROM Load/Dump Program image is

loaded into Boot Flash, use the following scenario.

a.  At the OPCON prompt (*):

1)  Enter TALK 5 to access the Monitor command environment (+).

* TALK 5

+

b.  At the MONITOR prompt (+):

1)  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

2)  You should see the following on the  Configuration  Information

screen:

Boot ROM Version  V2.20

The final step is to erase the PROM Load/Dump Program from the IBD where it

was stored earlier.

At  the  +  prompt  press  the CTRL and P keys simultaneously to get to the

OPCOM prompt (*).

+ CTRL C

*

At the OPCON prompt (*) enter TALK 6 to access the Config> command environ-

ment.

* TALK 6

Config>

At the CONFIG  prompt (Config>) enter BOOT to access the Boot  config  com-

mand environment.

Config> BOOT

Boot config>

1.  To  erase  the temporary copy of the PROM Load/Dump Program, prom_2.20,

enter ERASE and the filename at the "Boot config>" prompt.

Boot config> ERASE prom_2.20

The new version of the IBM 2210 PROM Load/Dump Program, 2.20, is now active

and loaded into Boot Flash.

LOADING THE PROM LOAD/DUMP PROGRAM USING BOOT MANUAL OVER THE LAN OR PPP

________________________________________________________________________

LINK

____

This scenario assumes that the IBM 2210 is connected via a LAN or PPP  link

to a PC or workstation running a TFTP server.

This  scenario  will load the PROM Load/Dump Program into DRAM and initiate

its execution.   The PROM Load/Dump Program  will  load  itself  into  Boot

Flash.

NOTE: YOU MUST ENTER THE RESPONSES THAT CORRESPOND TO YOUR ENVIRONMENT.

1.  Install  the  PROM Load/Dump Program into a PC or workstation running a

TFTP server that is reachable by the IBM 2210.   Ensure that  the  TFTP

server code is running before continuing.

The  following steps are performed while viewing the IBM 2210 console via a

local or remote service port connection.

2.  At the OPCON prompt (*), enter RELOAD to initiate the reloading of  the

IBM 2210.

* RELOAD

Are you sure you want to reload the gateway? (Yes or [NO]): Y

3.  While the machine is reloading (---- appears on the console),

Press  the  "CTRL and C" keys simultaneously to terminate reloading

and put the IBM 2210 into Boot Monitor state (the (>)  prompt  will

be displayed).

--------- CTRL C

>

4.  At the (>) prompt, enter BM to initiate the Boot Manual process.

> BM

5.  The  following  dialogue  will load the new PROM Load/Dump Program into

DRAM and initiate its execution. The PROM Load/Dump Program  will  load

itself into Boot Flash

NOTE: YOU MUST SPECIFY THE PARAMETERS THAT MATCH YOUR ENVIRONMENT.

Device types available:

IBD

Ethernet

WAN

Device type: ETHERNET

LAN port  [1]: 1

Connector Type (AUI/RJ45/AUTO_CONFIG) [AUTO_CONFIG]: AUI

Interface IP address [ ]: 1.1.1.1

IP mask [FFFFFF00]: FFFFFF00

Boot from host: 1.1.1.101

Boot file name: /PROMCODE/PROM_2.20

Using Ethernet at (     0, 0).

Trying host 1.1.1.101

file /u/diagnostic/prom_2.20

loading

...........

Starting at 1200000

----- Downloading boot image to Oncard Flash -----

Downloading boot image, please wait ...

Writing 524288 bytes

Download Complete, resetting in 5 seconds

PROM Load/Dump Program * Revision:  2.20 *

Copyright  1985-1996 Proteon, Inc.  All rights reserved

......

Loading using IBD Load Image "MRS.XXX"     XXX is S/W Extension)

----------------

The new version of the IBM 2210 PROM Load/Dump Program, 2.20, is now active

and loaded into Boot Flash.

If  you  want to verify that the new PROM Load/Dump Program image is loaded

into Boot Flash, use the following scenario.

1.  At the OPCON prompt (*):

a.  Enter TALK 5 to access the Monitor command environment (+).

* TALK 5

+

2.  At the MONITOR prompt (+):

a.  Enter CONFIGURATION to display the configuration.

+ CONFIGURATION

b.  You should see  the  following  on  the  Configuration  Information

screen:

Boot ROM Version  V2.20

printed from source file  "prom220 script" on 7/7/97.