Memorandum To: Licensees of IBM Software

Subject: Installing the MRS 2210 code.

This load module is provided as a complete load module replacement.

This  module  should  be installed on your server in a directory which

clearly defines the contents as MRS V1R1.0 PTF (eg. NP00737).

Before proceeding with this installation, contact IBM Software Service

and ask that they read or send you any special considerations noted in

the Preventive Service Package (PSP)  facility  for  upgrade  5765B86,

subset  MRS110.   If, after applying the 2210 load module, you experi-

ence problems, please contact IBM Software Support.

USING DISKETTES IN YOUR 2210 LOADING PROCESS

You may skip this section if you do NOT use diskettes in loading  your

2210 or if you use diskettes but the 2210 load WILL FIT one diskette.

If you are using diskettes during the process of loading your 2210 AND

the  load module is too large for one diskette, you will need the spe-

cial "cut" and "concatenate" utilities.   These utilities are  on  the

tools directory on the Internet.

1.  For use in an AIX environment:

KOPY_AIX     A  script that reads a 2210 load module from multiple

diskettes and concatenates the components to create a

2210 load module.

CUT_AIX      An executable that splits a 2210 module into multiple

components.  The components can then be  loaded  onto

diskettes.

2.  For use in other UNIX  environments:

KOP_UNIX     A  script that reads a 2210 load module from multiple

diskettes and concatenates the components to create a

2210 load module.

CUT_UNIX.C   UNIX C source file  that  can  be  compiled  using  a

standard  C  compiler and used to split a 2210 module

into multiple components. The components can then  be

loaded onto diskettes.

3.  For use in the DOS or OS/2 environments:

KOPY_DOS.BAT A  script that reads a 2210 load module from multiple

diskettes and concatenates the components to create a

2210 load module.

CUT_DOS.EXE  An executable that splits a 2210 module into multiple

components.  The components can then be  loaded  onto

diskettes.

The  following describes how to use the above utilities in the differ-

ent environments.

1.  AIX environments

a.  Build a 2210 load module from multiple diskettes:

o   The command syntax:

KOPY_AIX <OUTPUT PTF LOAD MODULE NAME> <NUMBER OF INPUT DISKETTES>

<OUTPUT DIRECTORY PATH (DEFAULTS TO CURRENT DIRECTORY)>

o   For example, the command

KOPY_AIX MRS.XXX 3 PTFSDIR

will read in 3  diskettes  and  build  2210  load  module,

MRS.xxx,  in the ptfsdir directory (off the current direc-

tory).

b.  Split the 2210 load module into multiple components so it  can

be loaded onto multiple diskettes:

o   The command syntax:

CUT_AIX <FULL PATH AND NAME OF 2210 LOAD MODULE>

<OUTPUT MODULE PREFIX> <OUTPUT MODULE SIZE>:

o   For example, the command

CUT_AIX /PTFS/MRS.XXX MRNS 1000000

will  split  2210  load  module  /ptfs/MRS.xxx  (length of

1500000) and create two files, mrns.0 (length of  1000000)

and mrns.1 (length of 500000)

2.  Other UNIX environments

a.  Build  a  2210  load  module from multiple diskettes using the

bourne shell script kop_unix:

o   Create the mount for the diskette  drive  and  destination

dictionaries

o   The command syntax:

KOP_UNIX <INSTALLATION DRIVE> <DESTINATION DIRECTORY>

<DISKETTE DIRECTORY> <OUTPUT PTF LOAD MODULE NAME>

<NUMBER OF INPUT DISKETTES>

o   For example, the command

KOP_UNIX /DEV/FD0 /KEW /PCFS MRS.XXX 3

will  read  in  3  diskettes  and  build 2210 load module,

MRS.xxx.

b.  Build a 2210 load module manually using the  following  proce-

dure:

o   Copy  the  components of the 2210 load module (mrns.0 from

diskette 1 and mrns.1 from diskette 2) into a directory on

the UNIX file system.

o   Type the following UNIX command

CAT MRNS.0  MRNS.1 > MRS.XXX

The resulting file (MRS.xxx) is the assembled router  load

module.

c.  Split  the  2210  load module so it can be loaded onto multipl

diskettes:

o   Compile the C program cut_unix.c to create cut_unix

o   The command syntax:

CUT_UNIX <FULL PATH AND NAME OF 2210 LOAD MODULE>

<OUTPUT MODULE PREFIX> <OUTPUT MODULE SIZE>

o   For example, the command

CUT_UNIX /PTFS/MRS.XXX MRNS 1000000

will split  2210  load  module  /ptfs/MRS.xxx  (length  of

1500000)  and create two files, mrns.0 (length of 1000000)

and mrns.1 (length of 500000)

3.  DOS and OS/2 environments

a.  Build a 2210 load module from multiple diskettes:

o   The command syntax:

KOPY_DOS <INPUT DISKETT DRIVE> <OUTPUT PTF LOAD MODULE NAME>

<NUMBER OF INPUT DISKETTES>

<OUTPUT DIRECTORY(DEFAULTS TO CURRENT DIRECTORY)>

o   For example, the command

KOPY_DOS A: MRS.XXX 3 PTFSDIR

will read in 3  diskettes  and  build  2210  load  module,

MRS.xxx,  in  the ptfsdir directory off of the current di-

rectory.

b.  Split the 2210  load  module  so  it  can  be  loaded  onto  2

diskettes:

o   The command syntax:

CUT_DOS <FULL PATH AND NAME OF 2210 LOAD MODULE>

<OUTPUT MODULE PREFIX> OUTPUT MODULE SIZE>:

o   For example, the command

CUT_DOS /PTFS/MRS.XXX MRNS 1000000

will  split  2210  load  module  /ptfs/MRS.xxx  (length of

1500000) and create two files, mrns.0 (length of  1000000)

and mrns.1 (length of 500000)

PTF INSTALLATION STEPS

The  following  steps describe how to install the new load module from

the server into the IBD (A description of  the  installation  is  also

provided  in the Nways Multiprotocol Routing Network Services Software

User's Guide).

1.  Install your load file into a server that is reachable by the  IBM

2210.  Make sure the TFTP daemon is running in your server.

2.  On  the  IBM 2210, issue the following commands at the router con-

sole:

a.  At the OPCON prompt (*):

1)  Enter TALK 6 :  to access the Config> command environment.

* TALK 6

Config>

b.  At the CONFIG  prompt (Config>):

1)  Enter BOOT.  to access the Boot config>  command  environ-

ment.

Config> BOOT

Boot config>

2)  At  the  Boot  config>  prompt, enter LIST IBD.  This will

show you what is already stored in the ibd.  Make sure you

have enough room for the new  load.    Each  storage  bank

holds 65536 bytes of data and your new load must be stored

in contiguous banks.

To erase an old load or configuration file enter ERASE

and  the filename.   Try to leave at least one load on

the IBD in case something goes wrong  in  loading  the

new load.

Boot config> ERASE filename

3)  At  the  Boot config> prompt, enter TFTP GET.  This initi-

ates the file transfer of the load module.

You will be prompted for the following information:

o   Local filename.   For the local  filename,  enter  the

filename   of   the   new   load   in  the  IBD,  e.g.

"ibd/MRS.AD0".  Note:  Because of an existing problem,

do not use a filename starting with a numeric  charac-

ter.

o   Remote  host.    For the remote host, enter the IP ad-

dress of the server.

o   Host filename.  For the host filename, specify the en-

tire path and filename on the host machine.

Boot config> TFTP GET

Local filename []?  IBD/FILENAME

Remote host []?

Host filename []?

4)  At the Boot config> prompt, enter LIST  BOOT-ENTRIES  This

lists the boot entries that are configured.

Boot config> LIST BOOT-ENTRIES

The boot database is used by the router to determine where

to  obtain  its load.  The router always tries to load the

first boot entry, then works its way down the list.

The first entry is usually a load module in the  IBD,  and

the  second  is  usually a load module on a remote host or

router.

If there are no boot entries listed, or  if  you  want  to

keep the ones already there, then you must add a new entry

for the new load module (remember that the loads are tried

in the order they appear in the database).

5)  From  the Boot config> prompt, enter ADD BOOT-ENTRY to add

the name of the new load module to the list.  You will  be

prompted for the filename of the new entry.

Boot config> ADD BOOT-ENTRY

Boot config> remote host or IBD loadname []?  FILENAME

6)  If  there are boot entries already existing, you will want

to change the boot database pointer to the module you just

loaded.

At the Boot config> prompt, enter CHANGE BOOT.

Boot config> CHANGE BOOT

You will then be prompted for  the  entry  number  you

want to change.  Since you want the router to use this

load to boot from you will choose entry "1".

Change which entry[] 1

Enter  the filename of the new load.  This is the name

that  you  specified  when  storing  into   the   IBD.

Filenames are case sensitive.

7)  Enter CTRL-P to return to the OPCON prompt (*).

8)  Enter  RESTART  to make sure the configuration change from

the "change boot" command takes effect.

* RESTART

9)  Enter RELOAD to load the router with the new load module.

* RELOAD

c.  Once you are confident with the new load, you can create space

in your IBD for future loads by erasing the previous load:

1)  Enter TALK 6.

* TALK 6

2)  Enter BOOT.

Config> BOOT

3)  Enter LIST IBD to list the content of the banks.  Note the

number of the banks where the previous load is stored.

Boot config>LIST IBD

4)  Enter ERASE and the filename of the load.  For example,

Boot config> ERASE FILENAME

Sincerely,

Service Delivery Manager