README.DOC file

MCS-350 & MCS-600/700 INSTALLATION

-------------------

Product Description

-------------------

The MCS-350/600/700 has been designed to provide easy "Plug and Play"

installation.

The MCS-350/600/700 supports all SCSI disk drives that conform to the

ANSI standard "Common Command Set". The MCS-350/600/700 ROM BIOS is

designed to identify and properly support the type of SCSI disk

drive it is installed with. The MCS-350/600/700 will work with IBM

PS/2 MicroChannel family and compatible computers.

---------------------

Physical Installation

---------------------

Follow the IBM instructions for removing the cover and inserting

the MCS-350/600/700 in your MicroChannel computer.

The MCS-350 external connection to a SCSI device is via the 25-pin

D-SUB connector at the head of the board.  Various cables and

adapters are available from Future Domain to convert 50-pin header

to 25-pin, 25-pin to 50-pin Amphenol, and cables for device

interconnect.

The MCS-700 external connection to a SCSI device is via the high

density connector at the head of the board.

The MCS-600 external connection to a SCSI device is via the 25-pin

D-SUB connector with Apple SCSI Pin-Out at the head of the board.

The HCA-120 cable must be used with this connector.

Contact Future Domain for assistance as to interconnect cables to

fit your application.

--------------------

System Configuration

--------------------

Boot up the system using the "Reference Diskette" that came with your

MicroChannel computer.  Make a copy of the diskette using the

"Backup the Reference Diskette" option from the main menu.

Next, copy the appropriate "ADF" file from the Future Domain

MCS-350/600/700 "Options Diskette" to the backup copy of "Reference

Diskette".  This allows configuration of the MCS-350/600/700 SCSI

adapter via the Reference Diskette's setup utilities.

For the MCS-350 board, you will use the "ADF" file @5F77.ADF.

For the MCS-600/700 (1800 based) board, you will use the "ADF" file

@6127.ADF. For the MCS-600/700 (18c50 based) board, you will use

the "ADF" file @60E9.ADF.

Once completed, boot from the backup copy of Reference Diskette

and select the "Set Configuration" option.  Follow the instructions

for setting up adapters in the MicroChannel System.  The MCS-350/600/700

will show up on the menu.

Note : The "Automatic Configuration" option may not always correctly

resolve system resource conflicts. Therefore, we recommend users

selecting the "Set Configuration" option instead.

For assistance or suggestions, please call or write to:

Technical Support

Future Domain Corporation

2801 McGaw Ave.

Irvine, CA 92714

Phone: (714) 253-0440

Fax: (714) 253-0429